| Pet Car Alarm: Challenge #1 | Goals: |
| --- | --- |
| In the Pet Car Alarm project, you will create a system that detects if the pet is in the car and measure the temperature. The car will sound a warning with flashing lights and roll down the windows if your pet could be in danger. In this challenge, you will use the TI-Innovator to create various notes by playing different frequencies. You will use these skills when sounding the alarm in your Pet Car Alarm. | - Use the SET SOUND command to play various notes. - Use a For loop to repeat code. |
| Background: |  |
| Did you know that all musical tones are the result of various frequencies? The middle C note on a piano has a frequency of 261.64 Hz. The C one octave higher is 523.28 Hz while one octove lower is 130.82 Hz. There are 12 notes in a octive. Each note can be calculated by taking the frequancy of the previous note and multiplying by the 122. Therefore, the note directly above middle C would be 261.64 × 122 = 277.198Hz. B is the note directly below C. What frequency would produce a B? ___________ Command; Example; Behavior. Wait <number>; Wait 3; Pauses the program for a specified number of seconds. In this case 3 seconds.. For(<counter variable>,<start value>,<end value>,[<step value>]) <statements> End; For(n,1,10) Output(3,1,n) End; Runs For loop 10 times, starting at 1 and ending at 10. Executes the statement in the block each time, displays the value for the counter variable on row 3.. SET(<type><number> ); Send(“SET SOUND 261.64 TIME 1”); Plays a 261.64 Hz frequency for 1 second. F:= 261.64 Send(“SET SOUND eval(F*2) TIME 1”); Sets the variable F to 261.64. eval() evaluates the value of the F*2 then plays the frequency for 1 second. If you use a variable for the frequency you must use eval().. |
| Challenge: |  |
| Use the Send(“Set Sound”) command to determine: the lowest frequency you can set and still hear it___________ the highest frequency you can set___________ Challenge: Write a program that will play two sounds for 1 second each in a For loop that repeats five times. e.g. Send(“SET SOUND 220”) Extension: Write a program that will play through an entire octave. Can you use a loop with math so you don’t have to use 12 different “SET SOUND” commands?. |