| Overview: |  |
| --- | --- |

In this project, you will be challenged to build and program an example of a feedback and control system. This engineering principle is central to many industrial systems and consumer products. During the project, you will engage with fundamental concepts in programming, such as variables, loops, conditional statements, and Boolean operators.

The project includes a series of challenges that build the conceptual knowledge and skills needed for the final open-ended challenge.

Note: For programming commands for this and other STEM coding projects, you can always refer to the “Python Syntax Quick Reference” document.

| Goals: |  |  |
| --- | --- | --- |
| - Create and edit a Python program on the calculator. - Connect the Hub to the calculator and a sensor to the Hub. - Write Python programs that include many foundational programming concepts. - Build a simple feedback and control system. |

Supplies:

Fig. 1: Setup of Temperature Sensor and Innovator Hub

TI-84 Plus CE Python Graphing Calculator

- Unit to Unit Cable

- TI-Innovator Hub

- Temperature sensor (small blue square)

- Grove Cable (multi-colored cable)

- Chenille (“fuzzy”) Wire (pipe cleaners)

Setup Project:

You each code your own calculator. Working in groups of two or three, share the TI-Innovator Hub, temperature sensors and other equipment.

Note: Carefully push the wire through the holes on the temperature sensor board to create a ring. The sensing element is on the flat side of the sensor board. Trial and error maybe a good way to go here, as you attempt to determine the most effective ring design (see example in Fig. 1).

Fig. 2: TI website with 10-min of Code sample programs

You can learn the basics of creating, editing and running Python programs on the TI-84 Plus CE by following some of the Python on the TI-84 Plus CE Python 10 Minutes of Code lessons (link). Those lessons are a step-by step approach to introductory programming concepts. Unit 1 will especially help you become familiar with creating, editing, and running Python programs on the TI-84 Plus CE.

If you are unfamiliar with Mood Rings or how they work, do a quick search on the internet for resources describing how a mood ring works. You may want to begin with this YouTube video,  https://www.youtube.com/watch?v=gvGl51xOKTU&feature=youtu.be

The following “Challenges” will provide a guided approach to building up from basic coding skills to a more complex final program that will emulate the behaviors of a mood ring.

Python enables extended functions beyond the core Built-in set by importing modules. For the Digital Mood Ring project, from ti.hub import * will bring in some of the additions that you will need. The import statement and functions are available on the TI Hub menus.

The screen shots below show some of the modules and how to access them.

Fig. 3: Starting a new Program

Fig. 4: Selecting Program Type

Fig. 5: Hub Project with Modules

Press [prgm] and then press 2: Python App to start a new Python program (Fig. 3). Since we will be using the TI Innovator Hub throughout these challenges, click on Types and select 6: Hub Project (Fig. 4).       This will automatically import two of the commonly used modules (Fig. 5).

Part 1:  Mixing Colors

Challenge 1: The color LED is controlled with the statement color.rgb(red value, green value, blue value). Each value ranges from 0 to 255, where 0 is off, 255 is full-on, and 128 is moderately dim.

Fig. 6: MOOD01 with Hub and color module

Write a program named "MOOD01" that turns on the Hub's color LED to red. Can you change the code to make a dim red?

In addition to annotating a program, comments can be used to turn a statement on and off. For example, in the Challenge 1 program, pressing [2nd] and the number 3 can be used to turn on/off for the execution of color.rgb(25,0,0) or the color.rgb(2,0,0) statements.

Sample code for MOOD01:

Activity 1: Mixing Colors

Fig. 7: RGB Color Model

In challenge 1, you learned about the three primary colors (R-G-B) used in digital devices and light-based media. But color printers, for example, don’t use those colors. The ink in those is based on the   C-M-Y (cyan, magenta, yellow) color model and black. Your students can explore color mixing in more detail on the Britannica website.

What secondary color is created from mixing two primary colors?

What happens when all three overlap?

Fig. 8: Sleep command in time module

Challenge 2: Use the color mixing Activity 1 as a clue to figure out the R, G, and B values for the secondary colors of cyan, yellow, and magenta. An RGB color chart (link) might help you identify the correct RGB values for the secondary colors.

Write a program that displays all three. Use sleep(2) for  a two-second pause between colors. You can find it under the time module (See Fig. 8).

Sample code for this challenge can be found in MOOD02.8xv

Challenge 3: Use the color mixing activity 1 as a clue to figure out the R, G, and B value(s) for white.

Fig. 9: MOOD03 with Hub and color module

Write a program that displays white and two darker shades of white (grayer). Use sleep() for a pause between shades. What effect does the line color.rgb(0,0,0) have on the color of the LED?

A sample program is provided in MOOD03.

Activity 2:   Fig. 10 shows a mood ring along with the mood colors the ring's stone could make depending on the "mood" (finger temperature) of the wearer.

Fig. 10: Colors of the Mood Ring

Use your knowledge of color along with the previous activities to find the R, G, and B values for all of the colors listed.

Record the values in the table below. Use these as a reference in the next challenge program.

| Color | R | G | B |
| --- | --- | --- | --- |
| Red |  |  |  |
| Green |  |  |  |
| Blue |  |  |  |
| Black |  |  |  |
| Orange |  |  |  |
| White |  |  |  |
| Cyan |  |  |  |
| Yellow |  |  |  |
| Magenta |  |  |  |
| Grey |  |  |  |

Fig. 11: MOOD04 sample code

Challenge 4:   Use the R, G, and B values you recorded in the table from activity 2 to write a program to display each mood ring color on the color LED.

Again, you should use the sleep() command for a short pause between each mood color (1 or 2 seconds).

Sample program for Challenge 4

(this corresponds to MOOD04).

Now that you have learned some of the science, engineering and programming concepts of color needed to complete the project, you can move on to Part 2 to begin creating your mood ring.

Fig. 12: Sample Code for Mood Messages

Fig. 13: Sample Mood Messages

Part 2: The Mood Ring

Challenge 5:  Choose five mood messages from the list on the right (Fig. 13) and write a program to display them, each on a different line, of the TI-84 Plus CE Python display.

Sample code is shown in Fig. 12 and provided under MOOD05.8xv

Challenge 6:   Combine your favorite mood color with your favorite mood message.

Write a program that displays both. You will find the ‘color’ module under the Hub module, 1: Hub Built-in devices …. (Fig. 14). Once you have imported that module, it will be available with the functions shown in Fig. 15. Sample code (MOOD06) is provided in Fig. 16.

What will happen as a result of the last line of code, color.rgb(0,0,0)?

Fig. 14: Hub Color Module

Fig. 15: Color Module options

Fig. 16: Sample Mood Messages

Fig. 17: Temperature Sensor in IN1

Challenge 7:    Connect the temperature sensor to IN1 (Fig. 17) and write a program that creates a temperature object named my_temp.

The temperature module can be found under the Hub module as well, 2: Input devices … Remember that the input port you choose in your code (in this case IN1) must match where the sensor is actually plugged in.

Once you have imported the module, it will be available with the functions shown in Fig. 18. Measure the my_temp object and store value in the variable named ‘temp’.

Fig. 19: Sample Code for Challenge 7

Fig. 18: Input Devices, 4: Temperature

Display an appropriate prompt with the measurement value and units. Where on the TI-84 screen will the temperature measurement be displayed?

Challenge 8:  Write a program that measures the temperature continuously, using a for loop that will read and display ten temperature measurements. In the program editor, press Fns ..., arrow over to Ctl and select 4: for i in range (size). (See Fig. 20)

Fig. 20: Fns Menu, for loop

Fig. 21: Sample Code for Challenge 8

Use sleep(2) to pause for two seconds in each cycle.

Sample code for this challenge can be found in MOOD08.

Challenge 9:  Use a while loop to continuously monitor temperature. The ‘while not escape’ loop can be found in the ti_system module (see Fig. 22). On the TI-84 Plus CE the [clear] key acts as an ‘escape’ key. Note that all the statements inside the ‘while not escape’ loop are indented. Use the Tools menu to add or delete indents.

Include if conditional statements using appropriate temperatures (temperature ranges) to display the following text and LED color:

- "It is cool"		blue

- "It is just right"		green

- "It is too hot"		red

Fig. 22: While loop with ‘escape’ key

Fig. 23: Sample Code for Challenge 9

Part of the sample code for MOOD09 is provided in Fig. 23.

Final Challenge:  Now you are ready to use the skills from all of the previous challenges to design and code your mood ring. Your program should display mood messages and colors over a range of finger temperatures.

Fig. 24: File Manager to copy Program 09

Helpful tips:

- As a starting point, modify a copy of the previous program. Open File Manager and scroll down to the program, MOOD09 in this case, and select Manage (see Fig. 24). On the next screen, select 1:Replicate Program ... and rename the previous program to make a copy.

Fig. 25: Sample Code for the Final Challenge

Include at least five if conditional statements for the  intervals that change the mood color and message based on finger temperatures. You can look back at Fig.12 for some suggestions.

- Temperature intervals of about two degrees will help your ring respond to typical body temperature measurements.