| Overview: |  |
| --- | --- |

In this project, students build and program an introductory example of a feedback and control system. This engineering principle is central to many industrial systems and consumer products. During the project, students will engage with fundamental concepts in programming, such as variables, loops, conditional statements, and Boolean operators.

The project includes a series of challenges that build the conceptual knowledge and skills needed for the final open-ended challenge.

Note: For programming commands for this and other STEM coding projects, refer to the “Python Syntax Quick   Reference” document.

| Goals: |  |  |
| --- | --- | --- |
| - Students will: - Create and edit a Python program on the calculator. - Connect the Hub to the calculator and a sensor to the Hub. - Write Python programs that include many foundational programming concepts. - Build a simple feedback and control system. |

Supplies:

Fig. 1: Setup of Temperature Sensor and Innovator Hub

TI-84 Plus CE Python Graphing Calculator

- Unit to Unit Cable

- TI-Innovator Hub

- Temperature sensor (small blue square)

- Grove Cable (multi-colored cable)

- Chenille (“fuzzy”) Wire (pipe cleaners)

Setup Project:

Each student codes their own calculator. Students may work individually or in groups of two, three or four to share the TI-Innovator Hub, temperature sensor and other equipment.

Note: Students will need to push the wire through the holes on the temperature sensor board to create a ring. The sensing element is on the flat side of the sensor board. Expect (and encourage!) trial and error as students attempt to determine the most effective ring design (see example in Fig. 1).

Fig. 2: TI website with 10-min of Code sample programs

Learn the basics of creating, editing and running Python programs on the TI-84 Plus CE by following some of the Python on the TI-84 Plus CE Python 10 Minutes of Code lessons (link). Those lessons are a step-by step approach to introductory programming concepts. Unit 1 will especially help your students become familiar with creating, editing, and running Python programs on the TI-84 Plus CE.

If some students are unfamiliar with Mood Rings or how they work, have students do a quick search on the internet for resources describing how a mood ring works. You may want to begin with this YouTube video,  https://www.youtube.com/watch?v=gvGl51xOKTU&feature=youtu.be

The following “Challenges” will provide a guided approach to building up from basic coding skills to a more complex final program that will emulate the behaviors of a mood ring.

Python enables extended functions beyond the core built-in set by importing modules. For the Digital Mood Ring project, from ti.hub import * will bring in some of the additions that you will need. The import statement and functions are available on the TI Hub menus.

The screen shots below show some of the modules and how to access them.

Fig. 3: Starting a new Program

Fig. 4: Selecting Program Type

Fig. 5: Hub Project with Modules

Press [prgm] and then press 2:Python App  to start a new Python program (Fig. 3). Since we will be using the TI Innovator Hub throughout these challenges, click on Types and select 6: Hub Project (Fig. 4).       This will automatically import two of the commonly used modules (Fig. 5).

Part 1:  Mixing Colors

Challenge 1: The color LED is controlled with the statement color.rgb(red value, green value, blue value). Each value ranges from 0 to 255, where 0 is off, 255 is full-on, and 128 is moderately dim.

Write a program named "MOOD01" that turns on the Hub's color LED to red. Can you change the code to make a dim red?

In addition to annotating a program, comments can be used to turn a statement on and off. For example, in the Challenge 1 program, pressing [2nd] and the number 3 can be used to turn on/off for the execution of color.rgb(25,0,0) or the color.rgb(2,0,0) statements.

Fig. 6: MOOD01 with Hub and color module

from ti_hub import *

import color

#set the color LED to red

color.rgb(255,0,0)

# set the color LED to a dim red

# color.rgb(25,0,0)

# set the color LED to a very dim red

# color.rgb(2,0,0)

Sample code for MOOD01:

Activity 1: Mixing Colors

Fig. 7: RGB Color Model

In challenge 1, students learned about the three primary colors (R-G-B) used in digital devices and light-based media. But color printers, for example, don’t use those colors. The ink in those is based on the C-M-Y (cyan, magenta, yellow) color model and black. Your students can explore color mixing in more detail on the Britannica website.

What secondary color is created from mixing two primary colors?

What happens when all three overlap?

Challenge 2: Use the color mixing activity 1 as a clue to figure out the R, G, and B values for the secondary colors of cyan, yellow, and magenta. An RGB color chart (link) might help your students identify the correct RGB values for the secondary colors.

#Import the ti_hub module

from ti_hub import *

import color

#Enter values to make secondary colors

color.rgb(0,255,255)

sleep(2)

color.rgb(255,255,0)

sleep(2)

color.rgb(255,0,255)

sleep(2)

color.rgb(0,0,0)

Fig. 9: Sample MOOD02 code

Fig. 8: Sleep command in time module

Write a program that displays all three. Use sleep(2) for a two-second pause between colors. You can find it under the time module (see Fig. 8). Sample code for this challenge can be found in MOOD02 (Fig. 9).

Challenge 3: Use the color mixing activity 1 as a clue to figure out the R, G, and B value(s) for white.

Write a program that displays white and two darker shades of white (grayer). Use sleep() for a pause between shades. What effect does the line color.rgb(0,0,0) have on the color of the LED?

A TI-84 sample program is provided in MOOD03.

#Import the ti_hub module

from ti_hub import *

import color

color.rgb(255,255,255)

sleep(2)

color.rgb(55,55,55)

sleep(2)

color.rgb(5,5,5)

sleep(2)

color.rgb(0,0,0)

Fig. 10: MOOD03 with Hub and color module

Fig. 11: Colors of the Mood Ring

Activity 2: Fig. 11 shows a mood ring along with the mood colors the ring's stone could make depending on the "mood" (finger temperature) of the wearer.

Have students use their knowledge of color along with the previous activities to find the R, G, and B values for all of the colors listed.

Record the values in the table below. Use these as a reference in the next challenge program.

| Color | R | G | B |
| --- | --- | --- | --- |
| Red |  |  |  |
| Green |  |  |  |
| Blue |  |  |  |
| Black |  |  |  |
| Orange |  |  |  |
| White |  |  |  |
| Cyan |  |  |  |
| Yellow |  |  |  |
| Magenta |  |  |  |
| Grey |  |  |  |

Challenge 4:   Use the R, G, and B values you recorded in the table from activity 2 to write a program to display each mood ring color on the color LED.

Again, the students should use the sleep() command for a short pause between each mood color (1 or 2 seconds).

Sample program for Challenge 4 (this corresponds to MOOD04).

Fig. 12: MOOD04 sample code

#Import the ti_hub module

from ti_hub import *

import color

# rgb values determined in activity 4

color.rgb(255,0,0)

# sleep will pause the running program

sleep(2)

color.rgb(0,255,0)

sleep(2)

color.rgb(0,0,255)

sleep(2)

color.rgb(0,0,0)

sleep(2)

color.rgb(230,60,0)

sleep(2)

color.rgb(255,255,255)

sleep(2)

color.rgb(0,255,255)

sleep(2)

color.rgb(255,255,0)

sleep(2)

color.rgb(255,0,255)

sleep(2)

color.rgb(8,8,8)

sleep(2)

color.rgb(0,0,0)

Now that your students have learned the science, 	   engineering and programming concepts of color needed to complete the project, you can move on to begin creating your mood ring.

Part 2: The Mood Ring

Fig. 14: Sample Mood Messages

Fig. 13: Sample Code for Mood Messages

Challenge 5:  Choose five mood messages from the list on the right (Fig. 14) and write a program to display them, each one on a different line, of the TI-84 Plus CE Python display.   Sample code is shown in Fig. 13 and is provided under MOOD05.8xv

Challenge 6:   Combine your favorite mood color with your favorite mood message.

Write a program that displays both. You will find the ‘color’ module under the Hub module, 1: Hub Built-in devices …. (Fig. 15). Once you have imported that module, it will be available with the functions shown in Fig. 16. Sample code (MOOD06) is provided in Fig. 17.

Fig. 17: Sample Mood Messages

Fig. 16: Color Module options

Fig. 15: Hub Color Module

Ask students what will happen as a result of the last line of code, color.rgb(0,0,0)?

Fig. 18: Temperature Sensor in IN1

Challenge 7:    Connect the temperature sensor to IN1 (Fig. 18) and write a program that creates a temperature object named my_temp.

The temperature module can be found under the Hub module as well, 2: Input devices … Remind the students that the input port they choose in their code (in this case IN1) must match where the sensor is actually plugged in.

Fig. 19: Input Devices, 4: Temperature

Once you have imported the module, it will be available with the functions shown in Fig. 19. Measure the my_temp object and store value in the variable named ‘temp’.

Display an appropriate prompt with the measurement value and units. Check with students to make sure they understand where the temperature will be displayed on the screen.

from ti_hub import *

from ti_system import *

from temperat import *

my_temp=temperature("IN 1")

temp = my_temp.measurement()

disp_at(6,"Temperature= "+str(temp)+" C","center")

Fig. 20: Sample Code for Challenge 7

Challenge 8:  Write a program that measures the temperature continuously, using a for loop that will read and display ten temperature measurements. In the program editor, press Fns ... , arrow over to Ctl and select

4: for i in range (size). (See Fig. 21)

Use sleep(2) to pause for two seconds in each cycle. Sample code for this challenge can be found in MOOD08.

Fig. 21: Fns Menu, for loop

Fig. 22: Sample Code for Challenge 8

# Hub Project

from ti_system import *

from time import *

from temperat import *

#use a loop with a range of 10

my_temp=temperature("IN 1")

for i in range(10):

t=my_temp.measurement()

disp_at(9,"Temperature = "+str(t)+" C","center")

# the sleep method pauses the program.

sleep(2)

Challenge 9:  Use a while loop to continuously monitor temperature. The ‘while not escape’ loop can be found in the ti_system module (see Fig. 23). On the TI-84 Plus CE the [clear] key acts as an ‘escape’ key. Note that all the statements inside the ‘while not escape’ loop are indented. Use the Tools menu to add or delete indents.

Include if conditional statements using appropriate temperatures (temperature ranges) to display the following text and LED color:

- "It is cool"		blue

- "It is just right"		green

- "It is too hot"		red

Fig. 23: While loop with ‘escape’ key

Fig. 25: Sample Code for Challenge 9

while not escape():

t=my_temp.measurement()

disp_at(5,"Temperature = "+str(t)+" °C","center")

# use the if statements to make a decision

if t<20:

disp_at(8,"It is cool","left")

color.rgb(0,0,255)

if t>=20 and t <26:

disp_at(8,"It is just right","left")

color.rgb(0,255,0)

if t>=26:

disp_at(8,"It is hot","left")

color.rgb(255,0,0)

Fig. 24: Sample temperature ranges

Final Challenge:  Now you are ready to use the skills from all of the previous challenges to design and code your mood ring. Your program should display mood messages and colors over a range of finger temperatures.

Helpful tips:

Fig. 26: File Manager to copy Program 09

As a starting point, modify a copy of the previous program. Open File Manager and scroll down to the program, MOOD09 in this case and select Manage (Fig. 26). On the next screen, select 1:Replicate Program ... and rename the previous program to make a copy.

- Include at least five if conditional statements for the intervals that change the mood color and message based on finger temperatures. See Fig. 28 for some suggestions.

- Temperature intervals of about two degrees will help your ring respond to typical body temperature measurements.

Fig. 28: Sample Code for the Final Challenge

.

t = my_temp.measurement()

disp_at(6,"Temperature = " + str(t)+"°C","left")

if t<20:

disp_at(8,"I feel happy","left")

color.rgb(255,0,0)

if t>=20 and t<22:

disp_at(8,"I feel hopeful","left")

color.rgb(0,255,0)

if t>=22 and t<24:

disp_at(8,"I feel meh","left")

color.rgb(0,0,255)

if t>=24 and t<26:

disp_at(8,"I feel calm","left")

color.rgb(255,255,0)

if t>=26:

disp_at(8,"I feel chill","left")

color.rgb(255,0,255)

sleep(1)

color.rgb(0,0,0)

Fig. 27: Mood colors and Messages

Fig. 28 shows portions of the sample code for the final challenge, MOOD_FIN.8xv