Some Like it Tepid!

TI-84 Plus CE Python and TI-Innovator Hub™ with TI-SensorLink™

Project Overview: In this activity, students are challenged to design a system that informs the user if their coffee is too hot to drink. The system should notify the user of three states: Too hot to drink; too cold to drink; and just right to drink. Students may choose to notify the user with a particular color of the Innovator Hub’s onboard LED and/or a particular sound played on the built-in speaker. After designing and building the system, use the hot water and then ice water to test the system. Use the results of your tests to adjust the code to ensure the system works as expected.

Parts List for project:

| - TI-Innovator™ Hub - TI 84 Plus CE Python - TI-SensorLink™ Adapter - Vernier BTA SS Temperature Probe | - Coffee cup or styrofoam cup - Hot water - Ice in cubes - Instant coffee (optional) |
| --- | --- |

To start a new program, press [prgm] and then press 2: Python App to start a new Python program (Fig. 1). Since we will be using the TI -Innovator Hub throughout these challenges, click on Types and select 6: Hub Project (Fig. 2). This will automatically import two of the commonly used modules (Fig. 3).

Fig. 3: Hub Project with Modules

Fig. 1: Starting a new Program

Fig. 2: Selecting Program Type

Example Python Code for TI-84 Plus CE:

| - Code snippet to set the RGB color LED to yellow on the Hub: import color color.rgb(255,128,0). - Code snippet to set speaker on the Hub to 256 Hz for 10 seconds: import sound sound.tone(256,10). - Code snippet to display a message on the calculator: disp_at(8,”Just right”,”left”). - Code snippet to connect and read the temperature sensor: tempsensor=vernier(“IN 1”,”temperature”) t = tempsensor.measurement() disp_at(5,“Temperature = “ + str(t)+ "°C"). - Code snippet to read and display the temperature sensor continuously every second until the [ecs] key is pressed. from vernier import * tempsensor=vernier("IN 1","TEMPERATURE") while get_key() !=”esc”: **t = tempsensor.measurement() **disp _at(5,”Temperature = “ + str(t)+ "°C",”left”). - Code snippet to a make decision tree using the if-then statement: if t>65: disp _at(8,"It is too hot","left") color.rgb(255,0,0) sound.tone(512,10) if t>=50 and t<=65: disp _at(8,"Just right","left") color.rgb(0,255,0) sound.tone(256,10) if t<50: disp_at(8,"Too cool","left") color.rgb(0,0,255) sound.tone(1024,10) sleep(1). | - The screenshot below shows the required modules and the Vernier temperature sensor connected to Hub “IN 1.” Complete sample program: from ti_hub import * from ti_system import * import color import sound from vernier import * tempsensor=vernier("IN 1","TEMPERATURE") disp_at(10,”Press [clear] to quit”,”left”) while not escape(): t = tempsensor.measurement() disp_at(5,"Temperature = " + str(t)+"°C","left") if t>65: disp_at(8,"It is too hot","left") color.rgb(255,0,0) sound.tone(512,10) if t>=50 and t<=65: disp_at(8,"Just right","left") color.rgb(0,255,0) sound.tone(256,10) if t<50: disp_at(8,"Too cool","left") color.rgb(0,0,255) sound.tone(1024,10) sleep(1) color.rgb(0,0,0) |
| --- | --- |